Home Lab Documentation

Architecture, Configuration and Security

Johnny  |  March 2026  |

| Classification | Internal - Personal Use |
| --- | --- |
| Last updated | March 2026 |
| Author | Johnny |

# 1. Overview

This document covers the full architecture, configuration, and security posture of a self-hosted home lab built across three Dell OptiPlex machines. The lab provides network-level ad blocking, encrypted VPN routing for all LAN traffic, self-hosted password management, media serving, and remote access - all running on second-hand hardware.

The network is segmented behind a dedicated OPNsense firewall VM. All LAN traffic is routed through a Mullvad WireGuard VPN tunnel. Remote access is handled exclusively via Tailscale with no ports open to the internet. The design prioritises privacy, security, and maintainability.

## 1.1 Design Principles

- No ports exposed to the internet. All remote access goes through Tailscale.

- All LAN traffic exits through Mullvad WireGuard. ISP cannot see traffic content or destinations.

- Tailscale traffic bypasses the VPN tunnel so remote access always works regardless of VPN state.

- DNS is handled internally by Pi-hole. No DNS leaks to ISP or third parties.

- Services are isolated to purpose-built VMs. No service runs on a Proxmox host directly.

# 2. Hardware

## 2.1 OptiPlex 390 - Firewall Node

| Item | Detail |
| --- | --- |
| Role | Dedicated OPNsense firewall and router |
| CPU | Intel Core i5-2400 |
| RAM | 4GB DDR3 |
| NICs | Built-in Intel NIC (WAN) + Intel 82574L PCIe NIC (LAN) |
| WAN | DHCP from Sky router |
| LAN IP | 10.0.0.1 |
| Proxmox access | Tailscale only - no LAN management IP |

## 2.2 OptiPlex 7040 - Main Hypervisor

| Item | Detail |
| --- | --- |
| Role | Primary Proxmox node - runs all current service VMs |
| CPU | Intel Core i5-6500 |
| RAM | 8GB DDR4 |
| Storage | 256GB SSD + 3TB external USB drive (NAS) |
| NIC | Built-in Intel NIC on vmbr0 |
| Proxmox IP | 10.0.0.181 |
| Tailscale | Yes |

## 2.3 OptiPlex 7050 - Media Node (Planned)

| Item | Detail |
| --- | --- |
| Role | Dedicated Proxmox node for Jellyfin media server |
| CPU | Intel Core i5-7500 |
| RAM | TBD |
| Storage | TBD |
| Proxmox IP | TBD - will be assigned static lease via OPNsense |
| Tailscale | Yes |
| Status | Hardware acquired - setup pending |

# 3. Network Architecture

## 3.1 Physical Layout

The Sky router provides the WAN connection and connects into the built-in NIC on the OptiPlex 390. OPNsense treats this as its WAN interface. The PCIe Intel NIC on the 390 connects to the unmanaged switch, which all other devices connect through. OPNsense is the sole gateway between the internet and the rest of the network.

The OptiPlex 7040 and 7050 both connect to the switch as standard LAN clients, with their VMs routing through OPNsense like any other device on the network.

## 3.2 IP Addressing

| Item | Detail |
| --- | --- |
| Subnet | 10.0.0.0/24 |
| Gateway | 10.0.0.1 (OPNsense LAN interface) |
| DHCP range | 10.0.0.41 to 10.0.0.245 |
| DNS for all clients | 10.0.0.157 (Pi-hole, pushed via Dnsmasq DHCP option 6) |
| OPNsense WAN | DHCP from Sky router (192.168.0.x range) |
| Mullvad tunnel address | 10.64.x.x/32 (from Mullvad config) |
| Tailscale subnet | 100.64.0.0/10 (WireGuard mesh, bypasses Mullvad) |

## 3.3 Traffic Flow

All LAN traffic follows this path by default:

Device on 10.0.0.x

--> OPNsense LAN (10.0.0.1)

--> WireGuard tunnel (wg0 / MULLVAD interface)

--> Mullvad VPN server (encrypted)

--> Internet

Tailscale traffic is exempted from the VPN tunnel via a firewall rule that routes 100.64.0.0/10 directly out through WAN_DHCP. This ensures remote access always works even if the Mullvad tunnel is down.

Tailscale traffic (dst 100.64.0.0/10)

--> OPNsense LAN

--> WAN_DHCP (bypasses Mullvad)

--> Tailscale control plane / peer

# 4. OPNsense Configuration

OPNsense runs as a VM on the OptiPlex 390 Proxmox node. The built-in NIC (eno1) is bridged as WAN via vmbr0. The PCIe Intel 82574L NIC (enp2s0) is bridged as LAN via vmbr1. Neither bridge has a host IP assigned on the 390 - Proxmox management on this node is via Tailscale only.

## 4.1 Proxmox Bridge Config

File: /etc/network/interfaces (OptiPlex 390)

auto lo

iface lo inet loopback

iface eno1 inet manual

auto vmbr0

iface vmbr0 inet manual

bridge-ports eno1

bridge-stp off

bridge-fd 0

auto vmbr1

iface vmbr1 inet manual

bridge-ports enp2s0

bridge-stp off

bridge-fd 0

Note: vmbr1 must have no host IP. Assigning one breaks OPNsense LAN routing.

## 4.2 WireGuard VPN - Mullvad

### Instance (VPN - WireGuard - Instances)

| Item | Detail |
| --- | --- |
| Name | mullvad |
| Private key | From Mullvad .conf file - PrivateKey field |
| Tunnel address | 10.64.x.x/32 - from Mullvad .conf file - Address field |
| Listen port | Leave blank |
| DNS | Leave blank - Pi-hole handles DNS |

### Peer (VPN - WireGuard - Peers)

| Item | Detail |
| --- | --- |
| Name | mullvad-peer |
| Public key | From Mullvad .conf file - PublicKey field under [Peer] |
| Allowed IPs | 0.0.0.0/0 |
| Endpoint address | IP portion of Endpoint field in .conf file |
| Endpoint port | 51820 |
| Instance | mullvad |

### Interface Assignment (Interfaces - Assignments)

Assign the wg0 WireGuard device as a new interface. Name it MULLVAD. Set IPv4 and IPv6 configuration type to None. Enable the interface.

### Gateway (System - Gateways - Configuration)

| Item | Detail |
| --- | --- |
| Name | MULLVAD_GW |
| Interface | MULLVAD |
| Address family | IPv4 |
| IP address | 10.64.0.1 |
| Far gateway | Enabled |
| Disable monitoring | Enabled |

## 4.3 Static Route

A static route is required to prevent a routing loop. Without it, OPNsense tries to route traffic destined for the Mullvad endpoint through the VPN tunnel itself, which prevents the tunnel from establishing.

| Item | Detail |
| --- | --- |
| Network | Mullvad endpoint IP/32 - e.g. 193.138.x.x/32 |
| Gateway | WAN_DHCP |
| Description | Mullvad endpoint bypass |

## 4.4 NAT Outbound

Switch outbound NAT mode to Manual. Add the following rule so LAN traffic is correctly masqueraded when exiting through the Mullvad interface.

| Item | Detail |
| --- | --- |
| Interface | MULLVAD (OPT1) |
| Source network | 10.0.0.0/24 |
| Translation target | Interface address |
| Description | VPN NAT |

## 4.5 Firewall Rules - LAN

Rules are evaluated top to bottom. The Tailscale bypass rule must sit above the VPN rule.

### Rule 1 - Tailscale bypass

| Item | Detail |
| --- | --- |
| Action | Pass |
| Interface | LAN |
| Protocol | Any |
| Source | LAN net |
| Destination | 100.64.0.0/10 |
| Gateway | WAN_DHCP |
| Description | Tailscale bypass |

### Rule 2 - Route all traffic through VPN

| Item | Detail |
| --- | --- |
| Action | Pass |
| Interface | LAN |
| Protocol | Any |
| Source | LAN net |
| Destination | Any |
| Gateway | MULLVAD_GW |
| Description | VPN all traffic |

## 4.6 DHCP and DNS

| Item | Detail |
| --- | --- |
| DHCP server | Dnsmasq |
| DHCP range | 10.0.0.41 to 10.0.0.245 |
| DNS pushed to clients | 10.0.0.157 (Pi-hole) via DHCP option 6 |
| OPNsense upstream DNS | 1.1.1.1 (used only if Pi-hole is unreachable) |
| GUI access | http://10.0.0.1 (port 80, LAN only) |

# 5. Services

## 5.1 Pi-hole - DNS and Ad Blocking

| Item | Detail |
| --- | --- |
| Host | Ubuntu VM on OptiPlex 7040 |
| IP | 10.0.0.157 (static via Netplan) |
| Web GUI | http://10.0.0.157/admin |
| Upstream DNS | 1.1.1.1 (Cloudflare) |
| Scope | All devices on 10.0.0.x receive Pi-hole as their DNS resolver |
| Service | pihole-FTL (system service) |
| Tailscale | Yes - also acts as subnet router |

## 5.2 Samba - NAS File Share

| Item | Detail |
| --- | --- |
| Host | Ubuntu VM on OptiPlex 7040 (same VM as Pi-hole) |
| IP | 10.0.0.157 |
| Share path | /mnt/nas |
| Drive | 1TB USB drive, mounted via fstab by UUID |
| Protocol | SMB/CIFS |
| Port | 445 |
| Boot note | nofail and x-systemd.device-timeout=30 in fstab prevents boot failure if USB drive is slow to mount |

## 5.3 Vaultwarden - Password Manager

| Item | Detail |
| --- | --- |
| Host | Dedicated VM on OptiPlex 7040 |
| IP | 10.0.0.167 (static via Netplan) |
| Local access | https://10.0.0.167 |
| Remote access | https://vaultwarden.tailb1a2c.ts.net (Tailscale) |
| Container | Docker: vaultwarden/server:latest (internal port 8080) |
| Reverse proxy | Caddy on port 443 |
| Local cert | Self-signed via OpenSSL |
| Remote cert | Tailscale certificate (tailscale cert command) |
| Mobile access | Bitwarden app - set server URL to Tailscale hostname with Tailscale active |

## 5.4 Uptime Kuma - Monitoring

| Item | Detail |
| --- | --- |
| Host | Monitoring VM on OptiPlex 7040 |
| IP | 10.0.0.194 (static via Netplan) |
| Web GUI | http://10.0.0.194:3001 |
| Remote access | Via Tailscale IP |
| Container | Docker: louislam/uptime-kuma:1 |
| Monitors | Proxmox 7040, Proxmox 390, Pi-hole, Ubuntu SSH, Samba port 445, Vaultwarden HTTPS, Tailscale control plane |

## 5.5 Jellyfin - Media Server (Planned)

| Item | Detail |
| --- | --- |
| Host | Dedicated VM on OptiPlex 7050 (setup pending) |
| IP | TBD - static assignment once 7050 is configured |
| Local access | http://[VM IP]:8096 |
| Remote access | Via Tailscale only |
| Port | 8096 |
| Status | 7050 hardware acquired - Proxmox and VM setup pending |

# 6. Tailscale

Tailscale provides the remote access layer for the entire lab. It runs on all Proxmox nodes and key service VMs. No ports are opened on OPNsense for Tailscale - it establishes outbound connections to the Tailscale coordination server and builds a peer-to-peer WireGuard mesh between devices.

Tailscale traffic is exempted from the Mullvad tunnel via the LAN firewall rule described in section 4.5. This ensures remote access works regardless of VPN state.

## 6.1 Nodes on Tailnet

| Item | Detail |
| --- | --- |
| OptiPlex 390 Proxmox | Management access only - no LAN IP on this node |
| OptiPlex 7040 Proxmox | 10.0.0.181 locally |
| Ubuntu VM (Pi-hole/Samba) | 10.0.0.157 locally - also subnet router |
| Vaultwarden VM | 10.0.0.167 locally - Tailscale cert issued |
| Monitoring VM | 10.0.0.194 locally |
| OptiPlex 7050 Proxmox | TBD - to be added when set up |
| Jellyfin VM | TBD - to be added when set up |

## 6.2 Subnet Router

The Ubuntu VM advertises the 10.0.0.0/24 subnet over Tailscale. This means any device connected to the tailnet can reach local services by their 10.0.0.x IP without needing Tailscale installed on each individual VM.

## 6.3 HTTPS Certificates

Tailscale HTTPS certificates are enabled in the Tailscale admin panel under DNS. The Vaultwarden VM has a certificate issued for its ts.net hostname, which allows the Bitwarden mobile app to trust the connection without certificate warnings.

Issue certificate:

sudo tailscale cert ████████████████████████████

# 7. Key Configurations

## 7.1 Ubuntu Static IP (Netplan)

File: /etc/netplan/00-installer-config.yaml

network:

ethernets:

ens18:

addresses:

- ██████████/24

routes:

- to: default

via: 10.0.0.1

nameservers:

addresses: [10.0.0.1]

dhcp4: false

version: 2

Apply:

sudo netplan apply

## 7.2 fstab - NAS Drive

File: /etc/fstab

UUID=YOUR-UUID  /mnt/nas  ntfs-3g  defaults,auto,users,rw,nofail,x-systemd.device-timeout=30  0  0

Get UUID:

sudo blkid /dev/sdb1

## 7.3 Caddy - Vaultwarden Reverse Proxy

File: /etc/caddy/Caddyfile

https://██████████ {

reverse_proxy localhost:8080

tls /etc/caddy/vaultwarden.crt /etc/caddy/vaultwarden.key

}

https://████████████████████████████ {

reverse_proxy localhost:8080

tls /etc/caddy/████████████████████████████.crt /etc/caddy/████████████████████████████.key

}

Restart Caddy:

sudo systemctl restart caddy

## 7.4 Tailscale Certificate Setup

Generate certificate:

sudo tailscale cert ████████████████████████████

Move and set permissions:

sudo mv ████████████████████████████.crt /etc/caddy/

sudo mv ████████████████████████████.key /etc/caddy/

sudo chown caddy:caddy /etc/caddy/████████████████████████████.crt

sudo chown caddy:caddy /etc/caddy/████████████████████████████.key

## 7.5 Vaultwarden Docker

sudo docker run -d \

--name vaultwarden \

--restart=always \

-v /vw-data/:/data/ \

-p 8080:80 \

-e DOMAIN=https://████████████████████████████ \

vaultwarden/server:latest

## 7.6 Uptime Kuma Docker

sudo docker run -d \

--name uptime-kuma \

--restart=always \

-p 3001:3001 \

-v uptime-kuma:/app/data \

louislam/uptime-kuma:1

# 8. Security Posture

This section documents the current security posture of the lab, including the protections in place, the remaining attack surface, and the risks that have not yet been mitigated.

## 8.1 What Is Protected

- No ports are open to the internet. OPNsense blocks all unsolicited inbound WAN traffic by default. There is no port forwarding configured.

- All LAN traffic exits through Mullvad WireGuard. The ISP cannot see traffic destinations or content. DNS queries go to Pi-hole and are not visible to the ISP.

- Remote access requires an active Tailscale session. Tailscale uses WireGuard underneath and requires authentication to join the tailnet.

- Vaultwarden is not exposed to the internet. It is only reachable locally or via an authenticated Tailscale connection.

- DNS-level ad and tracker blocking via Pi-hole reduces exposure to malicious domains across all devices on the network.

- OPNsense on the 390 has no local management IP. The only way to reach the Proxmox UI on that node is via Tailscale.

## ████████████████████████████████████████

██████████████████████████████████████████████████████████████████████████████

### ████████████████████████████████████████

███████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████

### ████████████████████████████████████████

█████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████

### ████████████████████████████████████████

█████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████

### ████████████████████████████████████████

████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████

### ████████████████████████████████████████

████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████

### ████████████████████████████████████████

██████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████

### ████████████████████████████████████████

███████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████████

## ████████████████████████████████████████

| Priority | Risk | Detail |
| --- | --- | --- |
| High | Tailscale account not protected by 2FA | Full tailnet access if account is compromised. This is the single most impactful control in the setup. |
| High | SSH password login enabled | Brute force possible on any VM for anyone on the local network. No fail2ban to rate limit attempts. |
| Medium | No VLAN segmentation | All devices on 10.0.0.x can reach all services. A compromised device has full internal access. |
| Medium | No VPN kill switch | If Mullvad tunnel drops, traffic may route through WAN directly without warning. |
| Medium | Sky router security | Default credentials or remote management enabled would expose the WAN-facing device. |
| Medium | Pi-hole admin over HTTP | Pi-hole panel accessible on plain HTTP. Anyone on the network could change DNS settings. |
| Low | Proxmox UI over self-signed HTTPS | Browser warnings may cause people to ignore certificate errors, normalising bad habits. |
| Low | No automated security updates | VMs may fall behind on patches without unattended-upgrades configured. |
| Low | USB NAS drive (single copy) | No backup of NAS data. Drive failure means permanent data loss. |

## ████████████████████████████████████████

████████████████████████████████████████

- ██████████████████████████████████████████████████████████████████████████

- ████████████████████████████████████████████████████████████████████

- █████████████████████████████████████████

- ███████████████████████████████████████████████████████████████████

- ██████████████████████████████████████████████████████████████████████████████████████████████

- ████████████████████████████████████████████████████████████████████████

- █████████████████████████████████████████████████████████████████████████████

- ██████████████████████████████████████████████████████████████████████████████████████████

# ████████████████████████████████████████

| VM | Host | IP | RAM | Services |
| --- | --- | --- | --- | --- |
| OPNsense VM | 390 | 10.0.0.1 | 1GB | Firewall, router, DHCP, WireGuard VPN |
| Ubuntu VM | 7040 | 10.0.0.157 | 2GB | Pi-hole, Samba, Tailscale subnet router |
| Monitoring VM | 7040 | 10.0.0.194 | 1GB | Uptime Kuma |
| Vaultwarden VM | 7040 | 10.0.0.167 | 512MB | Vaultwarden, Caddy |
| Jellyfin VM | 7050 | TBD | TBD | Jellyfin media server (planned) |

# 10. Future Plans

## Network

- VLAN segmentation - separate server, client, and IoT traffic

- WiFi access point connected to the switch, managed through OPNsense

- OPNsense Suricata IDS/IPS for intrusion detection

- VPN kill switch - block WAN traffic if Mullvad tunnel drops

- Pi-hole over HTTPS for encrypted upstream DNS

## Services

- Jellyfin on OptiPlex 7050 - accessible via Tailscale

- Central reverse proxy VM for all HTTPS services with proper certificates

- Homepage or Heimdall dashboard for all services

- Gitea for version-controlling all config files and documentation

- Immich for self-hosted photo backup

## Infrastructure

- Proxmox cluster across all three nodes for VM migration and high availability

- Proxmox Backup Server for automated VM snapshots

- Replace USB NAS with internal drives or a dedicated NAS VM

- UPS to protect against power cuts causing VM and filesystem corrupt